Dimensioning of riparian buffer zones in agricultural catchments at national level

Evelyn Uuemaa1, Ain Kull1, Kiira Mõisja1, Hanna-Ingrid Nurm1, Alexander Kmoch1

1Department of Geography, Institute of Ecology and Earth Sciences, University of Tartu, Vanemuise 46, 51003 Tartu, Estonia

Recommending optimal widths for riparian buffer zones to reduce the agricultural runoff is still a challenging task, in particular when considering the spatial variability of the landscape. Empirical-based approaches include assessment of terrain, soil types, land use and vegetation, and are often realised in computationally expensive hydrological simulation. However, trade-offs have to be made between spatial resolution and areal extent. Another elegant empirical-based approach are nomographs, where via triangulation of a specific slope length, terrain slope and soil type recommended buffer width can be easily calculated. Mander and Kuusemets (1998) already developed such a nomograph for Estonian catchments in 1998, yet, a computational use case has not been explored.

We implemented the nomograph as a GIS algorithm in Python/QGIS to retrieve the recommended buffer width at national level. As inputs we use specific slope length, slope (5m DEM) and soil texture classes. We synthesised the specific slope length via a weighted average of flow length, local flow accumulation and LS factor. We applied this algorithm and calculated recommendable buffer strip widths for the whole of country of Estonia, over an area of approx. 43,000 km2. We evaluated the uncertainty of the results as well the algorithm’s sensitivity to input weights.

The developed algorithm is applicable in any region with relevant adjustments to local soil types. The result directly informs policy making by being able to more specifically decide and explain variations of buffer zone widths along water bodies.