Nomo paper draft

Abstract

Recommending optimal widths for riparian buffer zones is still a challenging task, in particular when considering the spatial variability of the landscape. Empirical-based approaches include assessment of terrain, soil types, land use and vegetation, and are often realised in computationally expensive hydrological simulation. However, trade-offs have to be made between spatial resolution and areal extent. Another elegant empirical-based approach are nomographs, where via triangulation of a specific slope length, terrain slope and soil type recommended buffer width can be easily calculated. Mander and Kuusemets (1998) already developed such a nomograph for Estonian catchments in 1998, yet, a computational use case has not been explored.

We implemented the nomograph as a basic GIS algorithm to calculate the specific slopelength in order to retrieve the recommended buffer width. As inputs we use slope (5m DEM, Landboard 2019) and soil types (EstSoil-EH, 2021). We synthesisized the specific slopelength via a weighted average of flow length, slope length and LS factor, as can be easily calculated by any GIS. In the last step we digitised the defined nomogrpah angles and provide a lookup function. We applied this algorithm and calculated buffer strip widths for the whole of Estonia, over an area of approx 43,000 km2. We evaluated the uncertainty of the results as well the algorithms sensitivity to input weights.

This exercise directly informs policy making by being able to more specifically decide and explain variations of buffer zone widths along streams, rivers and all water protections zones in general based on a scientific location-specific analysis.

Introduction

riparian buffer zones serve as filter for surface and (very) shallow runoff from the landscape. In particular in regards to nutrient fluxes from agricultural fields, riparian buffer strips can mitigate degradation of river water quality. While widely acknowledging the effectiveness, much debate is still ongoing to agree on recommendable widths. This is partically due to either typically very localised studies on efficiency on one side with difficulties in upscaling to catchment or even regional scale. On the other side karger scale GIS-based assessments on pollution sensitivty draw generalised conclusions and don't take local landscape and flow patterns into account.

Hint: Andrew Fenemor literature review presentation on recommended buffer widhts HydSoc 2020

Hint: GIS/MCDA on pollution sensitivity in comparison can't fully explain a recommended buffer width?

To bridge this gap we are combining specific localised knowledge on buffer zone efficiency and scale it up to larger regions while embracing local variations such as flow patterns, variations in soil and terrain. In order to overcome computational burdens of nutrient flux simulation models we use empiracally-based nomograph function of Mander and Kusemets, which is validated on several Estonian catchments and captures the dynamics of nutrient fluxes on local scale for effective and specific recommendation of buffer strip widths. We take this nomograph and implement this as a raster-based GIS application. This pixel-based softare implementation can now be scaled at various solutions. GIS raster algebra calculations are computationally much less heavy than numerical model simulations, and have the advantage of being fully spatially distributed at the same time.

Materials and methods

Data and code folder: HPC-Rocket\kmoch\nomograph

Input datasets:

- EstSoil-EH, lxtype1 as USDA texture class for representing soil classes for the nomograph

Nomograph defined for soil classes:

7: coarse sand
6: fine sand
5: loamy sand
4: sandy loam
3: sandy clay - sandy loam
2: clay loam - sandy clay
1: loam

USDA texture classes in EstSOil-EH:

S - Sand
LS - Loamy Sand
SL - Sandy Loam
L - Loam
SiL - Silt Loam
SiCL - Silty Clay Loam
CL Clay Loam
C - Clay
HC - Heavy Clay
PEAT HUMUS
GRAVELS GRAVELS
no_info B no texture information

QGIS reclassification for Nomograph use:

WHEN "LXTYPE1" = 'GRAVELS' THEN 7

WHEN "LXTYPE1" = 'S' THEN 6

WHEN "LXTYPE1" = 'LS' THEN 5

WHEN "LXTYPE1" = 'SL' THEN 4

WHEN "LXTYPE1" = 'SiL' THEN 2
WHEN "LXTYPE1" = 'SiCL' THEN 2
WHEN "LXTYPE1" = 'CL' THEN 2

WHEN "LXTYPE1" = 'C' THEN 1
WHEN "LXTYPE1" = 'HC' THEN 1
WHEN "LXTYPE1" = 'L' THEN 1

WHEN "LXTYPE1" = 'PEAT' THEN 1
WHEN "LXTYPE1" = 'HUMUS' THEN 1

WHEN "LXTYPE1" = 'no_info' THEN 1
WHEN "LXTYPE1" = 'B' THEN 1

Discssion points:

- 'SiL' is Silty Loam and could even be class 1 or reinterpretation more sandy a 3

- 'C' and 'HC' clay could stay as 2

- Estonian Landboard Digital Elevation Model, 5m resolution Lidar

We derive several datasets from the DEM, including:

- slope in degrees

- LS Factor

- flow length (GRASS GIS)

- flow accumulation (GRASS GIS)

In order to make the computations for all of Estonia, we divided the processing area into 22 zones. The processing zones which have overlap with each other to ensure the spatial dependencies are maintained which are necessary for calculating flow path length and flow accumulation. For each zone we added a 5 km buffer in order to capture hydrological continuity across the zone boundaries. This way we make sure we don't create artificial interruptions along the flow direction paths. At the very end of the data processing the zones are clipped back to their original extent. The joined zones don't show any artifacts along their borders which confirms the effectiveness of the zoning method.

Discussion points:

We tested SAGA LS factor, SAGA slope length, SAGA flow accumulation, GRASS flow accumulation and GRASS flow length for 1m and 5m. Best result was GRASS flow length 5m which is quite similar to Wischmeier and Smith (1978) LS factor.

We also compare 1m and 5m GRASS slope length for final calc - 5m slope length is better

We validated several transect locations of the original nomograph data points and measured the slope length and flow accumulation on these slopes to determine the relationship between original nomogram slope length and GRASS slope length.

all three factors (flow length, flow accumulation and LS-factor) need to be normalised into 0 to 100 range. Normalisation formula

The three factors (flow length, flow accumulation and LS-factor) will be combined into one factor. All three values will be normalised to 0-100 and summed with equal weights into one factor in order to represent the specific slope length of the nomogram.

This formula will normalise into 0 to 1 which can be multiplied with 100 to convert into 0 to 100. To identify max and min values of each factor, the min and max values need to be checked across all 22 zones. It needs to be “global” min and max. (flowlen min=0, max=1391.504; flowacc min=0, max=21508, ls_factor is already normalized between 0-100).

In order to smoothen the effects of the different orders of magnitude within the factor inputs for the specific slope length calculation we apply a log transformation to to the input before the 0-100 normalisation. While initially intended for calibration, the weighing option has been kept constant (equal weights for all three inputs). It is later used for sensitivity analysis and providing means of uncertainty assessment.

Discussion points:

Weights between factors. Ain proposed 30%slope, 40% soil, 15% LS and 15% slope length between flow accumulation, flow length and ls factor. However, we kep equal weights to calculate the specific slope length.
The original flow accumulation scale of the nomograph is slightly misleadingly named as "specific slope length" with the intended concept being spatial water accumulation along the catchment’s flow paths) - the function "f(m)" was calibrated for the Porijõgi catchment?

Implementation as a QGIS processing algorithm:

The core of the digital nomogram is the relationship of the angles on the nomogram. These are implemented as Python NumPy trigonometric functions. With our data preprocessing we have now prepared per pixel inputs for the key variables:

- the specific slope length (based on the three input factors)

- the slope - angle in degrees

- the soil type, i.e. texture class

The tool loads the five input raster layers, which are provided in GeoTIFF format. It is crucial that the input raster are aligned in their origin point and resolution. The algorithm then applies the og transformation, the normalisation, the weighing of input layers, calculates the specific slope length and then calculates the recommendable buffer width based on the nomogram trigonmetric relationship function. As final output the algorithm writes out two raster layers with the same alignment and resolution as the input layers for the recommnedable buffer size as well as the calculated specific slopelength for reference.

Synthesising recommendable buffer strip widths along streams, rivers and water protection zones:

To better inform policy makers and catchment management we capture recommendable buffer zone widths along river and streams into a vector-based dataset extending the official river network data.

In GIS, geomorphology, catchment delineation it is a well-known fact that real-world rivers and streams do not always align perfectly with DEM derived flow lines. These may be artifacts in the digital elevation model or modifications of the river beds that are not (yey) reflected in the DEM.

We segmentize/subdivide the stream network into 5 meters long segments. Smaller resolution is not necessary as the raster resolution is also 5m. Then we define one-sided buffers along the river line segments of 50 meters each and perform zonal statistics for each stream segment buffer against the recommendable buffer widths raster dataset and take the median value to capture an acceptable local buffer width value, even if the stream bed defined by the vector layer does not exactly align with the hydrological dynamics derived from the DEM.

Is that correct?

Discussion points:

Why median? We needed to capture the dynamic of each local zone. The maximum value would likely be too harsh, and mean might be too influenced by outliers or a strong skew. Median captures robustly the local recommendable buffer width.
Also, general data quality issues, such as official water protection zones dataset has overlapping polygons and some zones do not align well with the streams.

Sensititvity and uncertainty analysis (for paper):

(not necessary for report? sufficient TODO document "Tsoonide statistikud lõplikule rasterkihile")

a) Nomograph uncertainty aka sensitivity of input data - vary weights systematically and calculate across Estonia (or selected representative zones for computation reduction), see which factors have strongest influence

b) Buffer uncertainty aka variations of buffer widths based on different weights – vary weights systematically and calculate across Estonia (or selected representative zones for computation reduction), see value distribution for buffer widths -> boxplots?

To perform the sensitivity analysis of weighing factors contributing to “Specific Slope Length f(m)” we apply a global sensitivity analysis called "Extended FAST" based on Saltelli et al. (1999). This is besides the Sobol sensitivity test the only algorithm available that is taking parameter interaction into account.

Eq: N = ( 1 + 4 x M**2) x ( 1 + ( k + 2 ) x d ) ) x k

with N = needed parameter iterations, M= inference factor (default M=4) and d= frequency step (default d=2) and k = 3 as the number of parameters of the model: N = 579

At least 579 simulation with varying weights for LS Factor, Flow Accumulation and Flow Length are recommended to assess and compare meaningfully the influence and ranking of each weighting parameter to a baseline equal weights setup; with 4 parameters (soil class choice) it would be 1284 simulations, with 5 parameters e.g. Estonian wide processing zone variation 2245 simulations, it might be interesting to the see variations in correlations, sensitivities and uncertainties for the different zones:

FAST Sensitivity for Processing Zone 21, hilly landscape in South-East strong influence of Flowlength weight factor, which has the strongest impact on the buffer width in this terrain.

FLOWLENWEIGHT: 0.72
FLOWACCWEIGHT: 0.31
LSFACTORWEIGHT: 0.45

FAST Sensitivity (above) for Processing Zone 7, rather flat landscape in North-West strong influence of LS factor weight

FLOWLENWEIGHT: 0.11
FLOWACCWEIGHT: 0.29
LSFACTORWEIGHT: 0.95

Discussion points:

For flow accumulation of the GRASS GIS algorithm for all 22 processing zones the largest values in Estonia reach ca. 21000. To better capture the topographic effects for water flow behaviour in the landscape we are integrating flow length and ls factor values. These values are much smaller and not directly comparable within the nomograph function. Thus, we normalised all 3 values into 0-100, weight and average them and then need to scale up again. This reduces extreme values from numerical raster-based flow accumulation and increases attention to local hill slopes close to streams. We call this the "Nomograph Catchment Co-Efficient". Based on the value of 21000 for the max flow accumulation and the normalisation to 100 for the data integration with flow length and ls factor we are testing with the upscaling value of about 200 (21000 / 100 = 210). With this we reach max specific slop length values of up to 7400m (not with newer log) which the original nomograph together with the soil classification translates into buffer zone strip - "P(m)" - pixel values of maximum xxx meters (for very few regions) - TODO histogram value distribution, analogous to single run summary with equal weights.

I should redo it and have a more stream-lined setup with the latest version of the nomograph algorithm (soil class mixup fixed and log transofrmation included), defined zones and defined if only against clipped areas or overall (compare with the stats images in TODO document "Tsoonide statistikud lõplikule rasterkihile")

Box plots of: 1) MEANs in sim. buffer width values; 2) RMSE, MAE, MAD, MEAN and STD for errors (i.e. differences of sim. rasters to baseline buffer raster); 3) MIN, MAX values in sim. buffer rasters and MIN, MAX for errors (i.e. difference of sim. rasters to baseline buffer raster) (left to right) for complete zone 21 (unclipped)

It would of course be great to assess the parameter uncertainty and sensitivity analysis in association with the actual effective lengths/widths of buffer zones along existing streams not only against all calculated raster pixels, is that possible in a scripted way? On the HPC cluster we can easily run stuff with high memory and very highly parallelised, each instance of nomograph calculation needs about 7 GB of RAM, for additional analytical computations such as dataframe use for correlation should consider almost doubling this.

For such an analysis we should consider not to fix 3m as default minimal buffer zone

Minimal buffer zone should be added as parameter to the script/processing plugin

Results

figures, values, tables

datasets

QGIS processing plugin, also has minimal buffer zone setting as parameter

Discussion

Conclusion

Acknowledgements

KIK project/Environment investment ...

Uni Tartu HPC centre

H2020 MSCA