Recommending optimal widths for riparian buffer zones is still a challenging task, in particular when considering the spatial variability of the landscape. Empirical-based approaches include assessment of terrain, soil types, land use and vegetation, and are often realised in computationally expensive hydrological simulation. However, trade-offs have to be made between spatial resolution and areal extent. Another elegant empirical-based approach are nomographs, where via triangulation of a specific slope length, terrain slope and soil type recommended buffer width can be easily calculated. Mander and Kuusemets (1998) already developed such a nomograph for Estonian catchments in 1998, yet, a computational use case has not been explored.

We implemented the nomograph as a basic GIS algorithm to calculate the specific slopelength in order to retrieve the recommended buffer width. As inputs we use slope (5m DEM, Landboard 2019) and soil types (EstSoil-EH, 2021). We synthesisized the specific slopelength via a weighted average of flow length, slope length and LS factor, as can be easily calculated by any GIS. In the last step we digitised the defined nomogrpah angles and provide a lookup function. We applied this algorithm and calculated buffer strip widths for the whole of Estonia, over an area of approx 43,000 km2. We evaluated the uncertainty of the results as well the algorithms sensitivity to input weights.